Masterarbeit: IK_RL MVP - Problemanalyse

Masterarbeit 
Master of Science im Studiengang Maschinenbau/Produktentwicklung
an der Fakultät für Anlagen-, Energie- und Maschinensysteme 
der Technischen Hochschule Köln

Projektteilnehmer:		Vanya Aziz
Matrikel-Nr.:	11136876
Adresse:		Deutz-Kalker-Str. 118
	50679 Köln
	vaziz@th-koeln.de

Professor:		Prof. Dr. Anja Richert
Betreuer:	Dario Luipers, M.Sc.

Köln, 21.01.2020

Letzte Änderung:		31.01.2021

Inhalt

Masterarbeit: IK_RL MVP - Problemanalyse	I

Inhalt	2

1.	Problem: The output of the PPO agent is always the limit of the action-range (either max. or min).	3

1.1	Observations	3

1.2	Potential Solutions	3

# Problem: The output of the PPO agent is always the limit of the action-range (either max. or min).

## Observations

i)	The models that are not trained for long will sometimes have outputs that are not 
 	the limits. Later models will always output [0.25, 0.9, 0.55]  ([min., max., min.])

ii)	Is the EE actually going to the positions, that the agent gives?
 	( No, State 22.01.2021)
 	 YES, State 25.01.2021

iii)	In the beginning, it tries z-values > height of the table, so that it does not collide with
 	the table. It gets punished with a lesser (better) value. In later training phases, the 
 	good z-value in the beginning is ignored and it converges to 0.55 (min).

iv) 	Tries to follow object only when the object is spawned at y = -3. 
 	 The weights of the object positions are not learned correctly. The minimal 
 	variance in the object spawn position does not allow for effective learning.

v) 	Test with the gym environment "MountainCarContinuous-v0"
 	 The output is between -1 and 1 with activation functopn

## Potential Solutions

i)	The way the action-bound is defined in PandaRobotEnv_ is wrong (Dynamically cant be changed: Restore old value [-1, 1]
 Even if action_boundary is changed to [-1, -1, -1] and [1, 1, 1], the max. possible 
     reward is -2.8x, with actions [1, 1, -1] (max., max., min.)

ii)	Change activation function: Current: tanh
 	ReLu: Still prefers limits, convergences to reward -2.8x
 	 Softmax: Same, with more deviation

Sigmoid: Same, around -2.8x

iii)	i) + ii)
	Same as ii). Around -2.8x and [max., max., min].

iv) 	Actions are not interpreted correctly. Would explain, that y and z are constant with 
   	only variance in x, that also later vanished.
	Preprocess actions before transmitting them to the IK-algorithm

v)	Change network architecture (current 150x150). Unlikely to solve problem

Same: [0.25 0.9  0.55] and around -2.8x

vi) 	Change learning rate (current 5e-4)
 	 Changed to 5e-9: Not enough time to see any progress, but doesn't look 
 	     promising

vii)	The cumulative reward is considered in the SB-PPO implementation. Whenever the   
 	method reset() is activated, the reward during the steps so far is added and consid 
 	ered the final reward. (Unlikely)

viii)	Change the boundaries of the observation-space (currently [-100, 100])

ix) 	i) + viii)

x)	i) + ii) + viii)

xi) 	Do not punish for crash with environment. Do not terminate in case if attribute 
 	self._contact_with_env assumes value TRUE. Max. updates: 150

Observations:

a)	150 Updates: Output assumes limit values over proportionally: Always sets 
  		z = 0.55 (min.). Does not follow object, if it is spawned near y=0.

b)	15 Updates: Sometimes z value is not min, but most of the time. In this case, 
 		the reward is much bigger. The positions of the IK-point are more random with
 		an emphasis on the limits, chaotic min.-max.

c)	90 Updates: It does not follow the object when y=0. It has a tendency to move 
 		to the limits, for the z-coordinate only min. (0.55).

Doesn’t follow object at all, except when the deviation from original spawn 
 	    position is very large (y=-3)

xii)	Convert distance reward punishment to positive reward. Use 1.8 as a constant.
  	1.8 + distance  (distance negative in standard configuration)

Converges to [1, 0.9, 1]  (max., max., max)

The "flipping" of the reward signal causes the intervals flip too. Now 
 	it converges to the max. z-value of 1 instead of the min 0.55.
 	WHY????????????

13)	The punishment of the crash is not strong enough.

At = Discounted sum of Reward (return) – Baseline estimate (value function)

Discounted Reward (return) = k=0∞γkrt+k

The punishment for the crash must be multiplied by the number of max. allowable
    	IK-points, so that when robot doesn't crash, the cumulative discounted reward is still
	smaller then the crash punishment.

- At= δt+ (γλ)δt+1 + ... + ... + (γλ)T-t+1*δT-1

where δt = rt + γV(s+1) - V(st)

- δt:

- Tells us the benefit of the new state over the old state.

- δt is the reward of the current state plus the difference in the estimated value 
         of the new state and the current value.

- The value function is the output of the critic.

- γ is the discount factor.

14) 	Significantly reduce the length of the memory trajectory T, update  for each 
 	self._max_IK_point (5) steps.
       Currently: 4 nminibatches, 128 steps, 128/4 updates per minibatch.

15) 	If the value of self_max_IK_points is reached, the agent is punished, no matter 
where the EE is. That means , that it can evaluate a good state as a bad state.

1. Embed the max. allowable IK points into the network. Implement 
	     self._max_IK_points into the observation space.

2. Do not punish agent for reaching max. allowable points.

.

16)	13)+15)

17)	13)+14)+15)

14) with:

i)	gamma = 0.99
		ii)	n_steps = 5
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 5
		ix)	noptepochs = 4
		c)	cliprange = 0.2

18)	13)+14)+15) with  self._max_IK_points = 2. It has been observed in 17), that the 
 	number of successive z-value output for preventing a crash is 2. It is theorized that
 	eventually with max_IK_points set to 2, the punishment will not be the 
	distance punishment + the crash punishment.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 5
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 5
		ix)	noptepochs = 4
		c)	cliprange = 0.2

19) 18) but with self._max_IK_points = 1

20)	13)+14)+15) with  self._max_IK_points = 2.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 10
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 2
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Converges very quickly to [1, 0.9, 1] with reward -0.5

21)  20, but with network architecture [20, 20]

22) 13)+14)+15) with  self._max_IK_points = 2.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Converges very quickly to [1, 0.9, 1] with reward -0.5

23)	13)+14)+15) with  self._max_IK_points = 2.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 9
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 3
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Converges very quickly to [1, 0.9, 1] with reward -0.5

23)	13)+14)+15) with  self._max_IK_points = 2.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 10
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 2
 		ix) 	cliprange_vf=-1 (Deactivated)
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Converges very quickly to [1, 0.9, 1] with reward -0.5

24)	13)+14)+15) with  self._max_IK_points = 2. 
	Action bound for all dimensions: 	[-10, 10]

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
 		ix) 	cliprange_vf=-1 (Deactivated)
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Slowly converges to -0.6x, before the model breaks (crashes always)

Successively reduce the learning rate.

24)	13)+14)+15) with  self._max_IK_points = 2. 
	Action bound for all dimensions: 	[-10, 10]

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
 		ix) 	cliprange_vf= None
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Slowly converges to -0.6x, before the model breaks (crashes always)

Successively reduce the learning rate.

25)	13)+14)+15) with  self._max_IK_points = 2. 
	Action bound for all dimensions: 	[-10, 10]

Terminating condition: 	self._current_points > self._max_IK_points, so that the
	agent can define a trajectory with two points.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
 		ix) 	cliprange_vf= None
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Slowly converges to -0.6x, before the model breaks (crashes always)

Successively reduce the learning rate.

26) 	25) + viii)

self._observation_bound = 2

- Had no effect, but trained for a very long period of time:

It converges to pos [10, 10, -10] with reward -0.5xxx

27)	13)+14)+15) with  self._max_IK_points = 2, but with only one action output to test
 	the PPO-algorithms performance for one output

Train for at an hour to see where it converges.

Action bound:	[-5, 5]

Terminating condition: 	self._current_points > self._max_IK_points, so that the
	agent can define a trajectory with two points.

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
 		ix) 	cliprange_vf= None
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- There are always two very different x-values that yield the nearly same reward:

1. Pos:	[1.8684136, 0.540351977, 0.9] -> -0.533133013699358
		2. Pos:	[1.073297  , 0.540350798, 0.9] -> -0.5331330115404658

Test these two positions with the given y- and z- coordinates. Possible 
 		     explanation: 1.86 is not reached anyways, and second is out of range too.
 		    EE remains in the limits of its reach in both cases.

- More interesting case:

1. Pos: [0.73660743, 0.64496244546, 0.9] -> -0.5238734310131028
		2. Pos: [0.58614990, 0.64491214493, 0.9] -> -0.5096705854587619

The oisition changes for 15 cm, but the reward difference was only 
	             equivalent to 2 cm change.

- Another case:

1. Pos: [0.33447105, 0.61938881052, 0.9] -> -0.5481061615180619
		2. Pos: [0.53165830, 0.61938925686, 0.9] -> -0.5477186493034072

This issue arises because of the method self._termination() within the 
	     IK-Loop.
	     The last action the agent gives is never executed but terminated in the 
	     while-loops, resulting  in a very similar reward in the last action since the 
	     environment had not the opportunity to move the EE except for one simulaiton
	     step.
 	     It does not matter, with which operators self._current_IK_points and 
	     self._max_IK_points are compared!

28)	13)+14)+15) with  self._max_IK_points = 2, but with only one action output to test
 	the PPO-algorithms performance for one output
	Action bound:	[-5, 5]

Terminating condition: 	self._current_points >= self._max_IK_points, so that the
	agent can define a trajectory with two points. (Restored to old condition)

Termination condition within the while- and for-loops in method step() changed
 	to if self._contact_with_env:

14) with:

i)	gamma = 0.99
		ii)	n_steps = 2
		iii)	entropy_coef = 0.01
		iv)	learning_rate = 5e-4
 		v)	vf_coef = 0.5
		vi)	max_grad_norm = 0.5
		vii)	lam = 0.95
		viii)	n_minibatches = 1
 		ix) 	cliprange_vf= None
		ix)	noptepochs = 4
		c)	cliprange = 0.2

- Best possibe slution for the action [AGENT_X , self._goal_pos[1], 0.9]:
	  For 50 Episodes: -0.5173544785723742

- Agent average reward for last 50 episodes: -0.60 (SEE TENSORBAORD
	  CHART)

Average reward for normal control with 
 	      np.array(reward_liste).mean() for reward_liste[-50:] is : 
             - 0.5173544785723742
	   Policy entropy reduces over time, assumes negative value

29)	Like 28, trained for 1 h.

Training start: 	d
	Training end:		d