Masterarbeit: Iterations - Conclusions

Masterarbeit 
Master of Science im Studiengang Maschinenbau/Produktentwicklung
an der Fakultät für Anlagen-, Energie- und Maschinensysteme 
der Technischen Hochschule Köln

Projektteilnehmer:		Vanya Aziz
Matrikel-Nr.:	11136876
Adresse:		Deutz-Kalker-Str. 118
	50679 Köln
	vaziz@th-koeln.de

Professor:		Prof. Dr. Anja Richert
Betreuer:	Dario Luipers, M.Sc.

Köln, 01.02.2021

Letzte Änderung:		09.02.2021

Inhalt

Masterarbeit: IK_RL MVP - Problemanalyse	I

Inhalt	2

1.	Action Space and Bound	3

2.	Horizon and Minibatch Size	3

# Action Space and Bound

The configuration of solution 28 has been slightly changed such that the action 
boundary of the agent was limited by the interval [0.25, 1], which is the originally 
intended action bound.
A few minutes into the training showed that the agent gets stuck in the interval limits, i.e., the actions are either 0.25 or 1. 
Since the only configuration change was the action bound, it can be theorized that:

i)	The activations of the neurons are highly dependent on the values in the 
 		observation space, making it not possible to "normalize" the output in between
 		some arbitrary interval. The agent doesn't learn that fine control is rewarded or 
   		makes any difference, there the only actions it will output are either very posi- 
 		tive or very negative. The swaging in between an interval might be success-
 		ful, if it occurs at all layers including the linear input layer. 
 		(Or maybe only the observation space and the output layer must be 
		normalized)

ii)	Therefore it is recommended to make a test run with very negative and very  
  		positive boundaries  and see which action values are outputted in a certain 
   		configuration. The max. and min. action values will be the lowest and highest 
 		observed actions.
 		This might lead to the necessity of further decreasing the learning rate, since 
 		the first learning steps are "wasted" in the unnecessary large action space

# Horizon and Minibatch Size

Based on data collected and summarized in "Hyperparameter_Tuning_Summary_02_02", iterations 12, 11, 9, 7 and 6, following statements can be made:

i)	With regards to optimal convergence speed, the horizon must contain as many 
  		steps as there are IK points for the approximation of the trajectory

ii)	With constraint i), the general loss function and the value loss function are 
 		highly volatile, possibly causing unstable learning sessions, whereas a high 
   		horizon limit significantly reduces the convergence speed must "smoothens" 
 		the curve of the aforementioned functions with respect to the update number.

iii)	For low manual maintenance, i.e., to prevent loading a model checkpoint 
		because later models broke, it is highly recommended to use
 		1. A multiple of the max IK points for the minibatch sizes
		2. A multiple of the minibatch sizes for the horizon.